Datasheet

No.: FC2CDR0357-120 240-FC03-45°x35°

Version: 2025-08-19-01

| ITEM: FC2CDR0357 | INPUT VOLTAGE(V) 120 240 | LENS FC03 | LENS DEGREE(°)θ1/2 45°x35° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 3×2W |  |
|  |  | CONSUMPTION(W) |  | 120=6.0 240=6.0 |  |
|  |  | OPERATING CURRENT(MA) |  | 120=51 240=30 |  |
|  |  | POWER FACTOR(PF) |  | 0.89 |  |
|  |  | LUMINANCE(LM) |  | CW=398 WW=359 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 08 |  |
|  |  | IP RATING |  | 67 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| HOUSING | 12# Diecasting aluminum powder coated T=60-80μm |  |  |  |  |
| FRONT RING | Hard chromeplated Stainless steel SUS 316L# |  |  |  |  |
| LIGHT WINDOW | Tempered step glass.T=8mm |  |  |  |  |
| CABLE GLAND | IP68 PG-9 PA66 |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| PCB | Excellent heat conductivity aluminum coefficient of heat Conductivity≥2.0w/mk |  |  |  |  |
| LED DRIVER | Constant Voltage Input, Constant Current Output |  |  |  |  |
| LENS | OPTICS Reflector efficiency≥85% |  |  |  |  |
| OPERATING TEMPERATURE | -20˚ C~40˚ C |  |  |  |  |
| POWER CABLE | Outside of luminaire 120V/240V: H05RN-F 3×1.0mm², L=0.5m |  |  |  |  |
| RECESSED SLEEVE MATERIAL | PC/ABS ALLOYS |  |  |  |  |
| GLASS PRESSURE CAPACITY | 2,578kgs |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |
| IP | IP67 |  |  |  |  |
| WATER PROOF SOLUTION |  |  |  |  |  |

| ACCESSORIES(Optional) |  |
| --- | --- |
| SLEEVE Optional1 | Optional2 |

PHOTOMETRIC

CERTIFICATES