Datasheet

No.: C2H0457-120 240-C18-80°×60°

Version: 2025-08-26-01

| ITEM: C2H0457 | INPUT VOLTAGE(V) 120 240 | LENS C18 | LENS DEGREE(°)θ1/2 80°×60° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 4×1.5W |  |
|  |  | CONSUMPTION(W) |  | 120=7 240=7 |  |
|  |  | OPERATING CURRENT(MA) |  | 120=59 240=33 |  |
|  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  | LUMINANCE(LM) |  | 491 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 09 |  |
|  |  | IP RATING |  | 67 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| HOUSING | 12# Die-casting aluminum powder coated. T=60-80μm. |  |  |  |  |
| FRONT RING | Hard chromeplated Stainless steel SUS 316L# |  |  |  |  |
| LIGHT WINDOW | Tempered step glass.T=8mm |  |  |  |  |
| CABLE GLAND | IP67 copper an d nicked coated |  |  |  |  |
| GLASS PRESSURE CAPACITY | Full area=2,578kgs |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| PCB | Excellent heat conduvtivity aluminum, coefficient of heat Conductivity≥2.0w/mk |  |  |  |  |
| LED DRIVER | Constant Voltage Input, Constant Current Output |  |  |  |  |
| OPERATING TEMPERATURE | -20˚ C~40˚ C |  |  |  |  |
| POWER CABLE | Outside of luminaire 120V/240V: H05RN-F 3×1.0mm², L=0.5m |  |  |  |  |
| RECESSED SLEEVE MATERIAL | PC/ABS ALLOYS |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |
| IP | IP67 |  |  |  |  |
| WATER PROOF SOLUTION |  |  |  |  |  |

ACCESSORIES(Optional)

SLEEVE

PHOTOMETRIC

CERTIFICATES