Datasheet

No.: XB2EFR0128-120 240-B54-20°

Version: 2025-09-01-01

| ITEM: XB2EFR0128 | INPUT VOLTAGE(V) 120 240 |  | LENS B55 |  | LENS DEGREE(°)θ1/2 28° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LIGHT SOURCE |  | 1×10W |  |
|  |  |  |  | CONSUMPTION(W) |  | 120=11.7 240=11.7 |  |
|  |  |  |  | OPERATING CURRENT(MA) |  | 120=98 240=56 |  |
|  |  |  |  | POWER FACTOR(PF) |  | 0.903 |  |
|  |  |  |  | LUMINANCE(LM) |  | 911 |  |
|  |  |  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  |  |  | IK RATING |  | 09 |  |
|  |  |  |  | IP RATING |  | 67 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |  |  |
| HOUSING | 12# Die-casting aluminum powder coated T=60-80μm Adhesion of ISO class 1/ATM class 4B |  |  |  |  |  |  |
| FRONT COVER | Hard chrome-plated stainless steel SUS 316L# |  |  |  |  |  |  |
| LIGHT WINDOW | Tempered step glass.T=8mm |  |  |  |  |  |  |
| LED TYPE | COB |  |  |  |  |  |  |
| GLASS PRESSURE CAPACITY | Full Area= 2,621kgs |  |  |  |  |  |  |
| CABLE GLAND | IP68 PG-11 PA66 |  |  |  |  |  |  |
| GASKET | Silicon rubber molding shaped seal |  |  |  |  |  |  |
| LED DRIVER | Constant Voltage Input, Constant Current Output |  |  |  |  |  |  |
| OPERATING TEMPERATURE | -20˚ C~40˚ C |  |  |  |  |  |  |
| POWER CABLE | Outside of luminaire 120V/240V: H07RN-F 3×1.0mm², L=0.5m |  |  |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |  |  |
| RECESSED SLEEVE MATERIAL | PC/ABS ALLOYS |  |  |  |  |  |  |
| ACCESSORIES(Optional) |  |  |  |  |  |  |  |
| SLEEVE Optional1 |  | HONEYCOMB LOUVER |  |  |  |  |  |

WATER PROOF SOLUTION

PHOTOMETRIC

CERTIFICATES