Datasheet

No.: B3FKA0559-120 240-FB12B5-45°

Version: 2025-09-02-01

| ITEM: B3FKA0559 | INPUT VOLTAGE(V) 120 240 | LENS FB12B5 | LENS DEGREE(°)θ1/2 45° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 5×3.5W |  |
|  |  | CONSUMPTION(W) |  | 120=21.7 240=21.7 |  |
|  |  | OPERATING CURRENT(MA) |  | 120=181 240=94 |  |
|  |  | POWER FACTOR(PF) |  | 0.929 |  |
|  |  | LUMINANCE(LM) |  | 1324 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 06 |  |
|  |  | IP RATING |  | 66 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| HOUSING | 12# Die-casting aluminum finishing or powder painting T=60-80μm. Adhesion of ISO class 1/ASTM class 4B |  |  |  |  |
| LIGHT WINDOW | Tempered glass.T=3mm |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| LED DRIVER | Constant Voltage Input, Constant Current Output |  |  |  |  |
| CABLE GLAND | IP68 PG-9 PA66 |  |  |  |  |
| GASKET | Tooling shaped seal |  |  |  |  |
| POWER CABLE | Outside of luminaire 120V/240V: H05RN-F 3×1.0mm², L=2.0m |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |
| WATER PROOF SOLUTION |  |  |  |  |  |

| PAINTING COLOR |  |
| --- | --- |
| Regular | Secondary |

PHOTOMETRIC

CERTIFICATES