Datasheet

No.: B7AEX0269-DC 24-B67-38°

Version: 2025-09-02-01

| ITEM: B7AEX0269 | INPUT VOLTAGE(V) DC 24 | LENS B67 |  | LENS DEGREE(°)θ1/2 38° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LIGHT SOURCE |  | 2×14W |  |
|  |  |  | CONSUMPTION(W) |  | 30.6 |  |
|  |  |  | OPERATING CURRENT(MA) |  | 1278 |  |
|  |  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  |  | LUMINANCE(LM) |  | 1705×2 |  |
|  |  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  |  | IK RATING |  | 05 |  |
|  |  |  | IP RATING |  | 65 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |  |
| FRONT COVER&HOUSING | 12# Die-casting aluminum finishing or powder painting T=60-80μm. Adhesion of ISO class 1/ASTM class 4B |  |  |  |  |  |
| LENS | OPTICS lens, efficiency≥ 85% |  |  |  |  |  |
| LED TYPE | COB |  |  |  |  |  |
| LIGHT WINDOW | Tempered glass. T=3mm |  |  |  |  |  |
| CABLE GLAND | Nickel coated copper |  |  |  |  |  |
| GASKET | Tooling shaped seal |  |  |  |  |  |
| PCB | Excellent heat conductivity aluminum, coefficient of heat Conductivity≥ 2.0w/mk |  |  |  |  |  |
| LED DRIVER | Constant voltage input, constant current output |  |  |  |  |  |
| OPERATING TEMPERATURE | -20˚ C~40˚ C |  |  |  |  |  |
| POWER CABLE | Outside of luminaire 12V/24V: H05RN-F 2×1.0mm², L=0.4m |  |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |  |
| DIAGRAM OF INSTRUCTION |  |  |  |  |  |  |

| PAINTING COLOR |  |
| --- | --- |
| Regular | Secondary |

PHOTOMETRIC

CERTIFICATES