Datasheet

No.: B2RKB0657P-120 240-B33-20°

Version: 2025-11-10-03

| ITEM: B2RKB0657P | INPUT VOLTAGE(V) 120 240 | LENS B33 | LENS DEGREE(°)θ1/2 20° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 6×2W |  |
|  |  | CONSUMPTION(W) |  | 120=11.8 240=12.1 |  |
|  |  | OPERATING CURRENT(MA) |  | 120=100 240=57 |  |
|  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  | LUMINANCE(LM) |  | 1226 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 08 |  |
|  |  | IP RATING |  | 67 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| HOUSING | Extruded aluminum, powder coated |  |  |  |  |
| FRONT COVER | Hard chrome-plated stainess steel SUS 316L |  |  |  |  |
| LIGHT WINDOW | Tempered step glass. T=6mm |  |  |  |  |
| LENS | OPTICS lens.efficiency≥85% |  |  |  |  |
| CABLE GLAND | IP68 PG-9 PA66 |  |  |  |  |
| GASKET | Glass and housing are silicone sealed |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| OPERATING TEMPERATURE | -20˚ C~40˚ C |  |  |  |  |
| POWER CABLE | Outside of luminaire 120V/240V: H05RN-F 3×1.0mm², L=0.5m |  |  |  |  |
| MOUNTING SLEEVE | Powder Coated Aluminum |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |
| WATER PROOF SOLUTION |  |  |  |  |  |
| B. IP67 Jiffy quick plug/socket connector(Optional) IP-67 connector is as an Optional assembly and ensures that water vaporwill not enter into luminaires directly. |  |  |  |  |  |

ACCESSORIES(Optional)

SLEEVE

PHOTOMETRIC

CERTIFICATES