Datasheet

No.: C4A0159A-DC 24-C9-45°+35°

Version: 2025-11-18-02

| ITEM: C4A0159A | INPUT VOLTAGE(V) DC 24 | LENS C9 | LENS DEGREE(°)θ1/2 45°+35° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 1×3.6W |  |
|  |  | CONSUMPTION(W) |  | 5 |  |
|  |  | OPERATING CURRENT(MA) |  | 210 |  |
|  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  | LUMINANCE(LM) |  | CW=416 WW=316 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 08 |  |
|  |  | IP RATING |  | 68 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| FRONT COVER & HOUSING | Hard chromeplated molding shaped stainless steel SUS 316# |  |  |  |  |
| LIGHT WINDOW | Step Tempered glass. T=8mm |  |  |  |  |
| GASKET | Silicone Gasket |  |  |  |  |
| LENS | OPTICS lens, efficiency≥85% |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| PCB | Excellent heat conductivity aluminum.coefficient of heat Conductivity≥2.0w/mk |  |  |  |  |
| LED DRIVER | Constant Voltage Input, Constant Current Output |  |  |  |  |
| POWER CABLE OPTIONAL | Outside of luminaire H07RN-F 2×1.0mm2 with water-stopper, L=3m, 5m |  |  |  |  |
| GLASS PRESSURE CAPACITY | 8932kgs |  |  |  |  |
| OPERATING TEMPERATURE | -20˚C~40˚C |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |
| RECESSED SLEEEVE | 115# PC/ABS ALLOYS, with 304 stainless steel bottom plate |  |  |  |  |

WATER PROOF SOLUTION

A. Water-Stopper(Included) IP-68 water-stopper is added in the end of luminaires as standard assembly which is also an important system for water resistance. B. IP68 Jiffy quick plug/socket connector(Optional) IP-68 connector is as an Optional assembly and ensures that water vapor will not enter into luminaires directly.

PHOTOMETRIC

CERTIFICATES