Datasheet

No.: B5AF0159-DC 24-B73-30°

Version: 2025-11-18-02

| ITEM: B5AF0159 | INPUT VOLTAGE(V) DC 24 | LENS B73 | LENS DEGREE(°)θ1/2 30° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 1×3.6W |  |
|  |  | CONSUMPTION(W) |  | 5 |  |
|  |  | OPERATING CURRENT(MA) |  | 210 |  |
|  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  | LUMINANCE(LM) |  | CW=426 WW=322 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 06 |  |
|  |  | IP RATING |  | 68 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| FRONT COVER | Hard chromeplated molding shaped stainless steel SUS 316# |  |  |  |  |
| HOUSING | Hard chromeplated molding shaped stainless steel SUS 316# |  |  |  |  |
| LIGHT WINDOW | Step Tempered glass.T=8mm |  |  |  |  |
| WATERPROOF SEAL | EPDM gasket |  |  |  |  |
| LENS | Separate OPTICS lens efficiency≥85% |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| CABLE GLAND | IP68 M1 6a copper with nickel-coated |  |  |  |  |
| PCB | Excellent heat conductivity aluminum.coefficient of heat Conductivity≥2.0w/mk |  |  |  |  |
| LED DRIVER | Constant Voltage Input,Constant Current Output |  |  |  |  |
| POWER CABLE OPTIONAL | Outside of luminaire H07RN-F 2×1.0mm2 with water-stopper, L=1m, 3m, 5m |  |  |  |  |
| OPERATING TEMPERATURE | Temperature of water between -20˚C~40˚C less than 1 meter depth |  |  |  |  |
| DIMMING SUPPORT | Triac PWM Available 1-10V Dali Customized |  |  |  |  |
| WATER PROOF SOLUTION |  |  |  |  |  |
| A. Water-Stopper(Included) IP-68 water-stopper is added in the end of luminaires as standard assembly which is also an important system for water resistance. B. IP68 Jiffy quick plug/socket connector(Optional) IP-68 connector is as an Optional assembly and ensures that water vapor will not enter into luminaires directly. |  |  |  |  |  |

ADJUSTABLE BEAM DIRECTION

PHOTOMETRIC

CERTIFICATES